# Duggoff Tire model

# +

# Longitudinal Model, for vehicle speed profile

# +

# Lateral Model for vehicle yaw rate and side slip

Simulink Simulations Report

Brian Lesko, Graduate Researcher and Teaching Associate, Masters of mechanical engineering student

Autonomy in Vehicles ECE 5553  | Ohio State University | 2/20/2023

with use of Robust Controls of Mechatronic Systems by Dr. Levent Guvenc

clc; clear; close all

Contents

A. Tire force model

Dugoff tire force model

longitudinal force vs longitudinal slip

longitudinal force vs side slip angle for slip values s = 0.1,0.2,0.3,0.4,0.5

tire force ellipse

B. Longitudinal vehicle model

Nonlinear longitudinal vehicle model in Simulink

Coast down profile

Full brake profile

Headwind Simulation

Road Slope Simulation

C. Lateral vehicle model

Yaw velocity gain

Linear vs nonlinear vehicle model

Yaw rate comparison

Side slip angle comparison

# A. The Complicated Duffoff Tire Model

Duggoff Model

SIMULINK Diagram

### Model Parameters can be found and described in "getTireParameters()" in the functions section

%[M,eta_t,lambda_t,lambda_f,I_w,R_w,Meq,C_d,rho,A_f,g,C_rr,C_x,L,l_f,l_r,C_y,I_z,mu,Fzf,Fzr] = getTireParameters();

## 1. Tire forces vs slip ratio

%Alpha is held constant

[M,eta_t,lambda_t,lambda_f,I_w,R_w,Meq,C_d,rho,A_f,g,C_rr,C_x,L,l_f,l_r,C_y,I_z,mu,Fzf,Fzr] = getTireParameters();

s=[0:0.01:1]';

figure, hold on

for alpha = (0:2:10)*pi/180

%at = [a*ones(length(time))',time];

sim('tire_model');

%v1 = saveVehicle(Fx,Fy,s,a); plot(v1.Fx,v1.s);

plot(s,Fx,'r','linewidth',2)

plot(s,Fy,'b','linewidth',2)

end

xlabel('slip'),ylabel('Force (N)'), title('Fx and Fy Longitudinal & Lateral Tire Force vs Slip Ratio'), grid on

legend('longitudinal','lateral')

Fx1 = Fx; Fy1 = Fy;

## 2. Tire Forces vs side slip angle

close all

%Alpha is held constant

[M,eta_t,lambda_t,lambda_f,I_w,R_w,Meq,C_d,rho,A_f,g,C_rr,C_x,L,l_f,l_r,C_y,I_z,mu,Fzf,Fzr] = getTireParameters();

alpha=[0:.5:20]'*pi/180;

figure, hold on

for s = (0.1:.1:0.5)

%at = [a*ones(length(time))',time];

sim('tire_model');

%v1 = saveVehicle(Fx,Fy,s,a); plot(v1.Fx,v1.s);

plot(alpha*180/pi,Fx,'r','linewidth',2)

plot(alpha*180/pi,Fy,'b','linewidth',2)

end

xlabel('slip angle alpha'),ylabel('Force (N)'), title('Fx and Fy Longitudinal & Lateral Tire Force vs Slip angle'), grid on

legend('longitudinal','lateral')

Fx2 = Fx; Fy2 = Fy;

## 3. Force Ellipse

Demonstrating that Fx and Fy are inversely related

close all

figure

alpha = [-1000:1:1000]*pi/180;

for s = [-0.1 -0.2 -0.3 -0.4 -0.5 -0.6 -0.7 0 0.1 0.2 0.3 0.4 0.5 0.6 0.7]

sim('tire_model');

plot (Fx, Fy,'r','linewidth',2)

hold on

xlabel('Longitudnal force (N)');

ylabel( 'Lateral force (N) ')

end

# B. Longitudinal Vehicle Model

# building off the tire force model above

### Longitudinal Vehicle Model Top Level Diagram

The step signals are added to create the desired motor torque commands in the following situations where the velocity over time is taken to be the output of the simulation

### Vehicle Model Diagram

The duggoff tire model is the lowest level diagram which was reviewed in the previous section of this report

Overall, The model takes into account brake and motor torque commands where the vehicle moves straight only. The following sections will demonstrate simulation outputs for different scenarios. The Example numbers in the model suggest that the driver is pressing the gas pedal from time 0 to time 300 with a strength of 50 (motor torque command is 50). Next, at time 300, the driver presses the gas pedal harder (motor torque command is 100). Then, at time 400, the driver lets go of the gas pedal (motor torque command drops to 0) and simultaneously, the driver presses the brake (brake command increases from 0 to 100).

### The example simulation numbers and veloctiy profile

close all, figure

motorStart = 50;

motorStartTime = 0;

motorIncrease = 100;

motorIncreaseTime = 300;

motorDecrease = 100;

motorDecreaseTime = 400;

brakeTime = 400;

breakStrength = 100;

[M, eta_t, lambda_t, lambda_f, I_w, R_w, Meq, C_d, rho, A_f, g, C_rr, C_x, C_alpha,L, l_f, l_r, C_s, I_z, mu] = getLogitudinalParameters();

sim('longitudinal_model')

plot(time,V,'r','linewidth',2), xlabel('Time (s)'),ylabel('Velocity (m/s)'),title('Vehicle Speed over time')

Simulation scenario 2, step rise in motor torque, Tm, wait for speed to settle, second step rise in Tm

close all, figure

motorStart = 75;

motorStartTime = 0;

motorIncrease = 25;

motorIncreaseTime = 300;

motorDecrease = 0;

motorDecreaseTime = 0;

brakeTime = 0;

breakStrength = 0;

[M, eta_t, lambda_t, lambda_f, I_w, R_w, Meq, C_d, rho, A_f, g, C_rr, C_x, C_alpha,L, l_f, l_r, C_s, I_z, mu] = getLogitudinalParameters();

sim('longitudinal_model')

close all, figure

plot(time,V,'r','linewidth',2), xlabel('Time (s)'),ylabel('Velocity (m/s)'),title('Vehicle Speed over time')

### Plotting V/(Motor Torque)

close all, figure, hold on

K = 1;

T = 2;

plot(time,V./Tm,'r','linewidth',2)

plot(time,K./(T.*time + 1),'b','linewidth',2)

Find K and τ values from this simulated

response for the simple model V/Tm = Ke^(−τd*s) /  (τ*s + 1) with τd = 0 and compare the simulated output to this model

simplified V/Tm = K/(τ*s + 1)

### Coast Down Simulation

Simulation scenario 3, step rise in motor torque, wait for speed to settle, then decrease motor torque to zero to record the coast down velocity profile.

close all, figure

motorStart = 75;

motorStartTime = 0;

motorIncrease = 0;

motorIncreaseTime = 0;

motorDecrease = 75;

motorDecreaseTime = 300;

brakeTime = 0;

breakStrength = 0;

[M, eta_t, lambda_t, lambda_f, I_w, R_w, Meq, C_d, rho, A_f, g, C_rr, C_x, C_alpha,L, l_f, l_r, C_s, I_z, mu] = getLogitudinalParameters();

sim('longitudinal_model')

### Coast Down Response

close all, figure

hold on

extractFrom = 108000;

len = length(time);

plot(time,V,'r','linewidth',2)

plot(time(extractFrom:len),V(extractFrom:len),'b','linewidt',2)

p = polyfit(time(extractFrom:len),V(extractFrom:len),2);

x = time(extractFrom:len);

poly = p(1)*x.^2 + p(2)*x + p(3);

plot(x,poly,'g','linewidth',2)

legend('original data','coast down','polyfit')

xlabel('Time (s)'),ylabel('Velocity (m/s)'),title('Vehicle Speed over time')

The Polynomial Coefficients are

s = sprintf('y = (%.1f) x^2 + (%.1f) x + (%.1f)',p(1),p(2),p(3))

### Full Brake Simulation

Let Speed settle with a given Tm, then record the braking response

close all, figure

motorStart = 75;

motorStartTime = 0;

motorIncrease = 0;

motorIncreaseTime = 0;

motorDecrease = 75;

motorDecreaseTime = 300;

brakeTime = 300;

breakStrength = 100;

[M, eta_t, lambda_t, lambda_f, I_w, R_w, Meq, C_d, rho, A_f, g, C_rr, C_x, C_alpha,L, l_f, l_r, C_s, I_z, mu] = getLogitudinalParameters();

sim('longitudinal_model')

### Full Brake Response Plot

close all, figure

hold on

extractFrom = 108000;

len = length(time);

plot(time,V,'r','linewidth',2)

plot(time(extractFrom:len),V(extractFrom:len),'b','linewidt',2)

p = polyfit(time(extractFrom:len),V(extractFrom:len),2);

x = time(extractFrom:len);

poly = p(1)*x.^2 + p(2)*x + p(3);

plot(x,poly,'g','linewidth',2)

legend('Acceleration','Full Brake','polyfit')

xlabel('Time (s)'),ylabel('Velocity (m/s)'),title('Vehicle Speed over time')

The Polynomial Coefficients are

s = sprintf('y = (%.1f) x^2 + (%.1f) x + (%.1f)',p(1),p(2),p(3))

Speed Settles, then headwind suddenly increases

close all, figure

motorStart = 75;

motorStartTime = 0;

motorIncrease = 0;

motorIncreaseTime = 0;

motorDecrease = 0;

motorDecreaseTime = 0;

brakeTime = 0;

breakStrength = 0;

% Headwind increases to 10 at time 300

[M, eta_t, lambda_t, lambda_f, I_w, R_w, Meq, C_d, rho, A_f, g, C_rr, C_x, C_alpha,L, l_f, l_r, C_s, I_z, mu] = getLogitudinalParameters();

sim('longitudinal_model')

close all, figure

hold on

extractFrom = 108000;

len = length(time);

plot(time,V,'r','linewidth',2)

plot(time(extractFrom:len),V(extractFrom:len),'b','linewidt',2)

p = polyfit(time(extractFrom:len),V(extractFrom:len),2);

x = time(extractFrom:len);

poly = p(1)*x.^2 + p(2)*x + p(3);

plot(x,poly,'g','linewidth',2)

legend('Acceleration','Headwind Increases','polyfit')

xlabel('Time (s)'),ylabel('Velocity (m/s)'),title('Vehicle Speed over time')

The polyfit should be moved to only the decreasing velocity portion, but note that this resembles braking strength

Speed Settles, then the road slope increases

close all, figure

motorStart = 75;

motorStartTime = 0;

motorIncrease = 0;

motorIncreaseTime = 0;

motorDecrease = 0;

motorDecreaseTime = 0;

brakeTime = 0;

breakStrength = 0;

% Road slope increases at time = 300 to alpha = 5

% Headwind increases to 10 at time 300

[M, eta_t, lambda_t, lambda_f, I_w, R_w, Meq, C_d, rho, A_f, g, C_rr, C_x, C_alpha,L, l_f, l_r, C_s, I_z, mu] = getLogitudinalParameters();

sim('longitudinal_model')

close all, figure

hold on

extractFrom = 108000;

len = length(time);

plot(time,V,'r','linewidth',2)

plot(time(extractFrom:len),V(extractFrom:len),'b','linewidt',2)

p = polyfit(time(extractFrom:len),V(extractFrom:len),2);

x = time(extractFrom:len);

poly = p(1)*x.^2 + p(2)*x + p(3);

plot(x,poly,'g','linewidth',2)

legend('Acceleration','Road Slope Increases','polyfit')

xlabel('Time (s)'),ylabel('Velocity (m/s)'),title('Vehicle Speed over time')

# C. Lateral Simulations

Compute the understeer gradient for the given vehicle and plot the yaw velocity gain versus vehicle speed. Verify that this is an understeer vehicle. (10 points)

Vehicle Parameters are stored in

%[L,g,l_r,l_f,m,Fz,C_f,C_r] = getUndersteerVehicleParameters()

%[L,g,l_r,l_f,m,Fz,C_f,C_r] = getOversteerVehicleParameters()

Plotting the Vehicle's Yaw Velocity Gain vs speed from the gradient

close all, figure

plotUndersteerGradients()

Note that most vehicles are designed to be understeering for ease of control. The test vehicle included. Notice how the reverse gear steering is slightly oversteered, which makes control of reverse gear less intuitive.

## Lateral Dynamics Simulink Models

Investigate the nonlinear and linear single-track models provided in same Simulink diagram.

[L,g,Lr,Lf,Cf,Cr,Cs,m,J,mu,R,Vref,alphaf,a11,a12,a21,a22,b11,b12,b21,b22,e2] = getLateralParameters();

The Linear and Nonlienar Lateral models, top level diagram

The linear model used is

The nonlinear model used is

0

Step Steering Input on Lateral Models

Steering Input of 5.7 degrees at 1 second, model comparison

Run Simulation

sim('lateral_models')

Yaw Rate Comparison

close all, figure, hold on

plot(time,yawRateLin,'r','linewidth',2)

plot(time,yawRateNonLin,'b','linewidth',2)

legend('linear model','nonlinear model'), title('Lateral Dynamics Simulations'),xlabel('Time (s)'), ylabel('Yaw Rate (radians)')

Side Slip angle Comparison

close all, figure, hold on

plot(time,betaLin,'r','linewidth',2)

plot(time,betaNonLin,'b','linewidth',2)

legend('linear model','nonlinear model'), title('Lateral Dynamics Simulations'),xlabel('Time (s)'), ylabel('Beta degrees')

WHY IS IT BETA?

Yaw Disturbance comparison

For a Yaw moment disturbance at t = 1

Run Simulation

sim('lateral_models')

close all, figure, hold on

plot(time,yawRateLin,'r','linewidth',2)

plot(time,yawRateNonLin,'b','linewidth',2)

legend('linear model','nonlinear model'), title('Lateral Dynamics Simulations'),xlabel('Time (s)'), ylabel('Yaw Rate (radians)')

close all, figure, hold on

plot(time,betaLin,'r','linewidth',2)

plot(time,betaNonLin,'b','linewidth',2)

legend('linear model','nonlinear model'), title('Lateral Dynamics Simulations'),xlabel('Time (s)'), ylabel('Beta degrees')

Functions

save the simulation output

function vehicle = saveVehicle(Fx,Fy,s,a)

vehicle.Fx = Fx; % the longitudinal force in the direction of wheel travel

vehicle.Fy = Fy;

vehicle.s = s;

vehicle.a = a;

end

Retreive Parameters for the longitudinal model

function [M, eta_t, lambda_t, lambda_f, I_w, R_w, Meq, C_d, rho, A_f, g, C_rr, C_x, C_alpha,L, l_f, l_r, C_s, I_z, mu] = getLogitudinalParameters()

M=2000; % Mass of the vehicle[Kg]

eta_t=0.9; % Transmission Efficiency

lambda_t=1.0; % Gear Ratio

lambda_f=4.1; % Final Drive Ratio

I_w=1; % Inertia of the Wheel[kgm2]

R_w=0.3; % Wheel Radius[m]

Meq=0.1*M; % Equivalent Mass Factor

C_d=0.29; % Drag coefficient

rho=1.225; % Air density[kg/m3]

A_f=2.8;% Frontal area	[m2]

g=9.81;% [N/m2]

C_rr=0.015;%Rolling resistance coefficient

C_x=3e5; %Longitudinal Stiffness[N]

C_alpha = 1.5e5; % Cornering Stiffness for 1 tire [N/rad]

L=2.85;% Wheelbase[m]

l_f=1.3;% Distance from the center of gravity of the vehicle (CG) to the front axle [m]

l_r=1.55;% Distance from the center of gravity of the vehicle (CG) to the rear axle [m]

C_s=1.5e5;% Cornering Stiffness of Front and Rear Tires [N/rad]

I_z=3700;% Inertia moment around z axis J or Iz [kg/m2]

mu=0.7; % Road friction coefficient

end

function [M,eta_t,lambda_t,lambda_f,I_w,R_w,Meq,C_d,rho,A_f,g,C_rr,C_x,L,l_f,l_r,C_y,I_z,mu,Fzf,Fzr] = getTireParameters()

%Parameters

warning off

M=2000*9.81; % Mass of the vehicle[N]

eta_t=0.9; %Transmission Efficiency

lambda_t=1.0; %Gear Ratio

lambda_f=4.1; %Final Drive Ratio

I_w=1; %Inertia of the Wheel[kgm2]

R_w=0.3; %Wheel Radius[m]

Meq=0.1*M; %Equivalent Mass Factor

C_d=0.29; %Drag coefficient

rho=1.225; %Air density[kg/m3]

A_f=2.8;%Frontal area	[m2]

g=9.81;% [N/m2]

C_rr=0.015;%Rolling resistance coefficient

C_x=3e5; %Longitudinal Stiffness[N]

L=2.85;%Wheelbase[m]

l_f=1.3;%Distance from the center of gravity of the vehicle (CG) to the front axle [m]

l_r=1.55;%Distance from the center of gravity of the vehicle (CG) to the rear axle [m]

C_y=1.5e5;%Cornering Stiffness of Front and Rear Tires [N/rad]

I_z=3700;%Inertia moment around z axis J or Iz [kg/m2]

mu=0.7; % Road friction coefficient

Fzf=(l_r*M)/(2*L);% Vertical force for front tire

Fzr=(l_f*M)/(2*L);% Vertical force for rear tire

end

function [L,g,l_r,l_f,m,Fz,C_f,C_r] = getUndersteerVehicleParameters()

% Parameters

L=2.85; % Distance between the axles [m]

g=9.81; %

l_r = 1.55;% Distance from the center of gravity of the vehicle (CG) to the rear axle

l_f = 1.3; % Distance from the center of gravity of the vehicle (CG) to the front axle

m = 2000; % Mass of the vehicle [kg]

Fz = (m*g)/4; % Vertical force is 1/4 on each tire

C_f = 3e5; % Cornering Stiffness of Front Tires

C_r = 3e5; % Cornering Stiffness of Rear Tires

end

function [L,g,l_r,l_f,m,Fz,C_f,C_r] = getOversteerVehicleParameters()

%Parameters

L=2.85; % Distance between the axles [m]

g=9.81; %

l_r = 1.55;% Distance from the center of gravity of the vehicle (CG) to the rear axle

l_f = 1.3; % Distance from the center of gravity of the vehicle (CG) to the front axle

m = 2000; % Mass of the vehicle [kg]

Fz = (m*g)/4; % Vertical force is 1/4 on each tire

C_f = 3e5; % Cornering Stiffness of Front Tires

%Reduced rear cornering stiffness to make Kus < 0;

C_r = 2e5; % Cornering Stiffness of Rear Tires

end

function plotUndersteerGradients()

%Oversteer, Yaw Velocity

[L,g,l_r,l_f,m,Fz,C_f,C_r] = getOversteerVehicleParameters();

Kus = (l_r/C_f/L - l_f/C_r/L)*m*g;

v_critical = sqrt(-L*g/Kus);

v = 1:0.5:25; %velocity

Gyaw_O = v./(L+Kus*v.^2/g);

plot(v,Gyaw_O,'LineWidth',3), hold on

%Understeer, Yaw Velocity

[L,g,l_r,l_f,m,Fz,C_f,C_r] = getUndersteerVehicleParameters();

Kus = (l_r/C_f/L - l_f/C_r/L)*m*g;

v = 1:1:50; %velocity

Gyaw_U = v./(L+Kus*v.^2/g);

plot(v,Gyaw_U,'LineWidth',3)

%Nuetral Vehicle

Kus_nuetral = 0;

Gyaw_nuetral = v./(L+Kus_nuetral*v.^2/g);

plot(v,Gyaw_nuetral,'LineWidth',3)

%The given Vehicle

[L,g,l_r,l_f,m,Fz,C_f,C_r] = getUndersteerVehicleParameters();

Kus = (l_r/C_f/L - l_f/C_r/L)*m*g;

Gyaw = v./(L+Kus*v.^2/g);

plot(v,Gyaw,'LineWidth',3)

[L,g,l_r,l_f,m,Fz,C_f,C_r] = getUndersteerVehicleParameters();

Cf_r = C_r;

Cr_r = C_f;

Lf_r = l_r;

Lr_r = l_f;

Kus_reverse = (Lr_r/Cf_r/L - Lf_r/Cr_r/L)*m*g; %static yaw rate gain under steady state conditions (s=0) steering input*Kus = yaw rate, at ss only

Gyaw = v./(L+Kus_reverse*v.^2/g);

plot(v,Gyaw,'LineWidth',3)

xlabel('Velocity, v, (m/s)'),ylabel('Yaw Velocity Gain, Gyaw'),title('Yaw Velocity Gain vs Velocity'), hold on

legend('Intense Oversteering','Intense Understeering','Nuetral Steering','The Tested Vehicle','Reverse Gear Steering')

axis([0 50 0 20])

end

function [L,g,Lr,Lf,Cf,Cr,Cs,m,J,mu,R,Vref,alphaf,a11,a12,a21,a22,b11,b12,b21,b22,e2] = getLateralParameters()

%% Parameters

L=2.85; % Distance between the axles [m]

g=9.81; %

Lr = 1.55;% Distance from the center of gravity of the vehicle (CG) to the rear axle

Lf = 1.3; % Distance from the center of gravity of the vehicle (CG) to the front axle

Cf = 3e5; % Cornering Stiffness of Front Tires x2

Cr = 3e5; % Cornering Stiffness of Rear Tires x2

Cs = 1.5e5; % Cornering Stiffness

m = 2000; %Mass of the vehicle [kg]

J = 3700; %Yaw moment of Inertia

mu = 0.7; %Dry coefficient of Friction

R = 0.3; % Wheel radius

Vref = 20; % Constant vehicle velocity.

alphaf = 0.1; % Steering wheel angle rad/sec

%%

%Linear Parameters Calculation

a11 = -(Cr+Cf)/(m*Vref);

a12 = -1-((Cf*Lf-Cr*Lr)/(m*Vref^2));

a21 = (Lr*Cr-Lf*Cf)/J;

a22 = -((Cf*Lf^2)+(Cr*Lr^2))/(Vref*J);

b11 = Cf/(m*Vref);

b12 = Cr/(m*Vref); %delta_r parameter

b21 = Cf*Lf/J;

b22 = Cr*Lr/J; %delta_r parameter

e2 = 1/J; % For yaw moment term

end